ASSIGNMENT-3

| Feature | man | whatis |
| --- | --- | --- |
| Output | Detailed manual pages | One-line description |
| Purpose | Learn complete usage of command | Quick reference |
| Options &Examples | Yes | No |
| Usage | man <command> | whatis <command> |

- ls -l | tee filename.txt

ls -l → Lists files in long format.

| → Pipes the output of ls -l to the next command.

tee filename.txt → Writes the output into filename.txt and also displays it on the terminal.

- The tee command can capture the output of a command and save it to a log file while still displaying it on the terminal. This is especially useful for monitoring scripts, system commands, or program outputs in real time while keeping a record.

ls -l /var/log | tee system_log.txt

  ls -l /var/log → Lists files in /var/log directory.

  | tee system_log.txt → Saves the output to system_log.txt and also displays it on the terminal.

- Step 1: Download the Ubuntu ISO

- Go to the Ubuntu official website.

- Download the Ubuntu 25.04 LTS ISO file

### Step 2: Install Oracle VirtualBox

- Download VirtualBox from https://www.virtualbox.org/.

- Install it on your system (Windows, Mac, or Linux).

### Step 3: Create a New Virtual Machine

- Open VirtualBox → Click “New”.

- Enter:

  - Name: Ubuntu25.04

  - Type: Linux

  - Version: Ubuntu (64-bit)

- Click Next.

### Step 4: Allocate Memory (RAM)

- Recommended: 4096 MB (4 GB) or more.

- Click Next.

### Step 5: Create a Virtual Hard Disk

- Choose “Create a virtual hard disk now” → Click Create.

- Disk type: VDI (VirtualBox Disk Image) → Click Next.

- Storage on physical disk: Dynamically allocated → Click Next.

- Set disk size: 20 GB or more → Click Create.

### Step 6: Configure the Virtual Machine

- Select the VM → Click Settings.

- Go to Storage → Empty (under Controller: IDE).

- Click the CD icon → Choose a disk file → Select Ubuntu 25.04 ISO.

- Go to System → Processor → Enable 2 CPUs (optional for better performance)

### Step 7: Start the Virtual Machine

- Click Start.

- VirtualBox will boot from the Ubuntu ISO

### Step 8: Install Ubuntu

- Choose “Install Ubuntu”.

- Select Language, Keyboard Layout.

- Configure:

  - Normal Installation

  - Download updates while installing (optional)

- Choose Erase disk and install Ubuntu (Virtual disk only).

- Set Your Name, Username, Password.

- Click Install → Wait for the installation to complete

### Step 9: Restart the Virtual Machine

- After installation, click Restart Now.

- Remove the ISO from Storage → Controller: IDE

### Step 10: Log in and Start Using Ubuntu

- Enter your username and password.

- Ubuntu 25.04 LTS is ready to use on VirtualBox

### 5.

### Verify Installation Media

- Re-download ISO and check SHA256 checksum.

- Recreate bootable USB or virtual disk.

### 2. Check VM / Hardware Settings

- Ensure 64-bit OS, sufficient RAM (≥4 GB), 2+ CPU cores.

- Enable EFI if needed.

### 3. Adjust Boot Parameters

- At GRUB, edit boot line: add nomodeset, acpi=off, or noapic.

- Press F10 to boot.

### 4. Test Hardware

- Run memory test (Memtest86+).

- Check hard disk/SSD health.

### 5. Alternative Install

- Use Ubuntu Minimal or Server Edition.

- Try an older kernel if available.

### 6. Update Virtualization Software

- For VirtualBox: use latest version and install Guest Additions after boot

6.

- Display the hostname

hostname

hostname → Shows the current hostname of the system.

- Change hostname using sysctl

Change temporarily

sudo sysctl kernel.hostname=newname

- Make it permanent

sudo nano /etc/hostname

- Update /etc/hosts to reflect the new hostname

sudo nano /etc/hosts

- Reboot to apply permanently

sudo reboot

Example:
Change hostname to ubuntu-vm

sudo sysctl kernel.hostname=ubuntu-vm

hostname  # Verify change

- cal 8 1984

cal → Displays the calendar.

8 → Month (August).

1984 → Year.

Output

August 1984

Su Mo Tu We Th Fr Sa

1   2   3   4

5    6    7     8   9  10   11

12   13  14  15  16 17  18

19   20   21  22 23 24  25

26   27   28  29 30  31

- uptime; who

uptime → Shows how long the system has been running, load averages, and current time.

who → Shows the users currently logged in.

; → Runs both commands sequentially.

Output

21:45:12 up 5 days,  4:12,  3 users,  load average: 0.15, 0.10, 0.05

inchara   tty1         2025-09-27 20:15

root      pts/0        2025-09-27 20:20

guest     pts/1        2025-09-27 21:00

Can also combine into a single line using echo

echo "Uptime:"; uptime; echo "Logged-in users:"; who

- find /home/user -type f -name "*.c"

/home/user → Directory to start searching from.

-type f → Search for files only.

-name "*.c" → Match files ending with .c.

Output

/home/user/program.c

/home/user/projects/test.c

/home/user/notes/example.c

- We use chmod command

chmod 600 filename

Example

chmod 600 secret.txt

ls -l secret.txt

Output

-rw------- 1 user user 1024 Sep 27 21:50 secret.txt